Project Progress Report

Student Name: Sarah Brown

Project Title: cqunix-sysadmin

Date: June 1, 2025

Report Number: Weekly Report #1

- Summary of Progress This Week

- • Report started.

- • Project implementation plan including deadlines.

- • Router (rocky) is set up with NAT and internal network (“intnet”).

- • DHCP (darwin) is set up to assign IP addresses to two client machines. Connected via intnet.

- • Web server (adelaide) is set up and showing Apache home page. Connected via intnet.

- • SSH server (sydney) is set up to with keys (riley & sarah) from both client machines (WS01 & LP01). Connected via intnet.

- • Git server (gladstone) is set up with Git (with users riley & sarah), the same as in Week 11. Keys for SSH are on machine. Able to add ., commit, push and pull. Connected via intnet.

- • WS01 and LP01 (clients) with two users (riley & sarah) set up to connect to the internet via intnet, can connect to sydney and gladstone via SSH.

- Tasks in Progress

- • Configuring iptables on rocky.

- • Setting up DokuWiki for adelaide.

- Upcoming Tasks

- • Configure audit mechanism for SSH (sydney).

- • Sync DokuWiki via Git (gladstone).

- • Obtain cert for Web server (adelaide).

- • Write script for backup server (bundaberg).

- • Testing

- • Presentation

- • Finishing report

- • Second progress report

- Challenges / Issues

- • DHCP server assigned IP addresses to the two client machines outside of the range I assigned, regardless of what I did. WS01 was assigned the same IP address as sydney. Fixed by hardcopying MAC and IP in code.

- • Originally, rocky wouldn’t connect to the internet. Issue with NAT interface. I had to switch to Bridge Adaptor. All VMs are connected to the internet now.

- Reflection / Notes

- • A lot more time management than I first expected, but I helped myself by adding deadlines when I was outlining the project implementation plan.

- • Need to double check extension as I’m completing this as if it’s due on June 13.